MATHEMATICAL MODEL AND CALCULATION PROGRAM FOR THE PROCESS OF DESIGN AND DEVELOPMENT OF HYDROCARBON FIELDS

N.Kurbanov, S.Alibekov, S.Murodov
Abstract: The article considers the mathematical model of three-phase «oil-gas-water» mixture filtering in porous media, as well as software for the calculation of the design and development of hydrocarbon deposits. Proposed mathematical instrument can be used in determining the redistribution of pressure and saturations in the oil and gas deposits. That gives practical application in development of mathematical support for automated design of oil and gas deposits.

Keywords: mathematical model, numerical algorithm, software, gas wells, filtering, gas, oil, water.
Introduction.
Further growth of the oil-gas industry is closely related to development of methods for mathematical modeling of real objects and complex processes occurring in porous media. Currently, the most of oil and gas fields are being developed under artificial impact conditions, where the natural gas or water is used as an agent.
Improvement of existing and implementation of new methods of mathematical modeling is necessary for in-depth study of the mechanism of physical systems in which there is a moving mass of liquid and gas, and energy transfer from one species to another.
The study of many of these processes causes necessity of creation adequate mathematical models and computational algorithms to determine the dynamic state and control of reservoir filtration systems, taking into account the dynamics of transients. Recently, many researchers in the world have received considerable theoretical and practical results on considered issues 
In [1] there is a model of multicomponent filtration medium in which the relative permeability of the gas phase is replaced by a new expression that takes into account the effect of viscosity, density and capillary effect of the mixture.

The study of capillary pressures corresponding triangular capillary diffusion tensor in three-phase fluid was discussed in [2]. Filtering with such tensor is described by solutions for degenerate parabolic system of equations. This system is integrodifferential, as total flow and the distribution of phase saturations are sought in a given pressure drop in one of the phases at the boundaries of the flow domain. It was shown that the problem of displacement degenerating capillary system can be studied on the basis of a special maximum principle.

The problem of transport three-phase mixture "water-gas-oil" in a porous medium was considered in [3] in the case where the water contains fine gas phase in form of micro- or nano-scale bubbles. It is assumed that the transport of bubbles is mainly determined over a disperse phase (water). In this case, large accumulations of gas phase in the pore space, as well as water and oil carried in accordance with the modified Darcy law for multiphase mixtures. A mathematical model of the mixture motiontion, when the main phase (water, gas, oil) are subject to the equations of the filter, and the fine gas phase is described by the kinetic equation of the Boltzman.

The problem of filtration of three viscous, incompressible and mutually immiscible fluids in porous media without taking into account the mass forces and capillary pressures between phases was considered in [4]. There was obtained a solution for three-phase flow similar to solutions of Buckley-Leverett for two phases and it also was shown that the nature of the distribution of the saturations significantly depends on the initial saturation of the porous layer and the phase composition of the injected mixture. While developing the oil fields the problem of collaborative filtering several phases occurs.
In [5-8] was numerically solved and studied the problem of one-dimensional and three-phase filtration in a heterogeneous reservoir subject to solubility of the gas in the oil and aqueous phases, compressibility of  the phases and porous medium, and force of gravity.
The study of the problem of two-phase and three-phase fluid filtration was discussed in [7]. And in [8] in a more general way, subject to many features it was posed and solved the problem of filtration of multiphase mixtures. Currently, the process of joint motion of oil and water, oil, water and gas when phases moving is immiscible are fairly well understood. However the filtering multiphase mixtures with immiscible phases still isn’t studied sufficiently.
Mathematical models of multiphase fluid and gas in multilayer porous systems which are being developed are based on the general laws of mechanics and they reduce to systems of nonlinear differential equations with appropriate initial, boundary and internal conditions, characterized by a change of variable states of the system which are not possible to be solved analytically. Therefore, while mathematical modeling of seepage flows of multicomponent mixtures, authors use various simplifications in the physical formulation of the problem and thereby simplify the mathematical model of the object of study.
Main part.
To display the mathematical model of the filtering process of multicomponent media, considering the Darcy's law- which is the equation of continuity, phase state and the condition of equality of the sum of saturated mixtures unity, we will  obtain a closed system of nonlinear differential equations [9-11]:



 
[image: image1.wmf](

)

(

)

,

,

,

1.

KKpKPS

divKgradPmSS

RTZtRTZ

KK

divgradPmS

t

KK

divgradPmS

t

SSS

rr

rr

mmm

rr

m

rr

m

HHBBGG

HHBB

HBG

H

HHH

H

B

BBB

B

HBG

ì

éù

æö

éù

¶

æö

++=++

ï

êú

ç÷

êú

ç÷

¶

èø

ëû

èø

ï

ëû

ï

æö

¶

ï

=

ï

ç÷

¶

í

èø

ï

æö

¶

ï

=

ç÷

ï

¶

èø

ï

++=

ï

î

(1)

where - the index 
[image: image2.wmf]H

 indicates the phase of oil, 
[image: image3.wmf]G

 - gas and 
[image: image4.wmf]B

 - water; 
[image: image5.wmf]t

 - parameter of time; 
[image: image6.wmf]P

 - reservoir pressure; 
[image: image7.wmf],,

SSS

HBG

 - Saturation of rocks with oil, water and gas; 
[image: image8.wmf]m

 - porosity; 
[image: image9.wmf]K

 - absolute permeability of rocks; 
[image: image10.wmf],,

KKK

HBG

и 
[image: image11.wmf],,

mmm

HBG

 - correspondingly phase permeabilities and viscosity of oil, water and gas; 
[image: image12.wmf],,

rrr

HBG

 - corresponding density of of oil, water and gas; 
[image: image13.wmf]R

 - universal gas constant; 
[image: image14.wmf]T

 - absolute temperature; 
[image: image15.wmf]Z

 - overcompressibility factor.

For the numerical integration  the system (1), with using the relation 
[image: image16.wmf]***

2

,,,,

KRTZ

xyP

xyPt

LLP

mL

r

t

m

H

H

H

====

 we will translate to a dimensionless form.

With the given well flow rates 
[image: image17.wmf](

)

,

i

Э

qtiNN

H

Î

U

 (
[image: image18.wmf],

Э

NN

H

 - ets of injection and production wells, respectively), 
[image: image19.wmf](

)

0

,

ttT

Î

the functions of saturation and pressure distribution all over the area of productive layer can be found by solving the system of equations below


[image: image20.wmf](

)

(

)

(),

(),

(),

1.

H

H

H

P

KPSBSBS

xxtt

S

P

KB

xxt

S

P

KB

xxt

SSS

m

m

GHBHB

BB

B

H

BG

¶¶¶¶

ì

=++

ï

¶¶¶¶

ï

¶

¶¶

ï

=

ï

¶¶¶

í

ï

¶

¶¶

=

ï

¶¶¶

ï

ï

++=

î

 

(2)

with the initial, boundary and internal conditions for production and injection galleries:

[image: image21.wmf](

)

(

)

(

)

,0(),,0(),,0(),

PxPxSxSxSxSx

HHBB

===

ooo




(3)

[image: image22.wmf]00

,1,,

ii

жj

PP

Kqjn

xx

xx

+-

¶¶

æö

-==

ç÷

¶¶

èø






 (4)

[image: image23.wmf](

)

(

)

11

10,0,

жo

P

KqPPatx

x

aa

H

¶

éù

-+--==

êú

¶

ëû




 (5)


[image: image24.wmf](

)

(

)

212

10,1.

жn

P

KqPPatx

x

aa

+H

¶

éù

-+--==

êú

¶

ëû

 


(6)

The parameters in the boundary conditions (5) and (6) take the value 0 or 1.

The problem is solved this domain 
[image: image25.wmf](

)

(

)

{

}

,,:01,,1,,0

j

xtxtxxjnt

x

Î<<¹=>

.
To integrate the problem numerically there was developed conservative numerical algorithm based on combined use of integro-interpolation method and the sweep method [9-11]. На основе разработанного алгоритма создана программа расчёта для процессов проектирования и разработки углеводородных месторождений, состоящий из следующих программных модулей: управляющий модуль для ввода данных (Fig.2); произведения расчета на ЭВМ (Fig.1); анализ и интерпретация результатов проведенных численных расчетов (Fig.3-4).
On the basis of the algorithm, has been developed a program for the calculation of the design and development of hydrocarbon deposits, which is consisting of the following software modules: a control module for data entry (Fig.2); works for computing (Fig.1); analysis and interpretation of the results of the numerical calculations (Fig.3-4).
The program is designed for the analysis of the dynamic state of the object based on the joint motion of a three-phase medium under various conditions of the system in which the change in pressure and saturation fields in time and space depending on the hydrodynamic parameters of the object (Fig.1-4).

To analyze and study the filtration process "oil-gas-water" in porous media and identifying the results of key parameters of filtration processes and their range of variation of Computational experiments on a computer (Fig.4).
The program involves the interface mode for maximum visualized, of man-machine interaction.

After starting the display shows the main program window (Fig.1).

[image: image26.png]T Mporpaniia pacira 4n NpoUEcCoB MpoeKTAposaHA n paspaoTiyresoRopoHem Mectopoxcaes [ || B )
C————
Dara/Bpems 0 nporamme
19.11.2014 17:15:28 Nporpanma pactera s npoueccos

NDOEKTHDOBaHMS! 1 PazpaGoTKH
VI7ICBOAODOAHDIX MECTOPOXACHHH

=0 e i pasTHLIX YCOBHEX
Astopsi
H — sasegyrouymi naGopaTopnei, A.T.H,

Paswaros
H.M.KypOOHOB ~ M3 uumsi HayHbisi COTDYAHNK

[ Bomcnenme | [ i Boxon





fig.1. Main window of the program
The program allows you to maintain and change the necessary data in real mode for obtaining solution of the problem (Fig. 2).
[image: image27.png]B Buoasenmc

on. nofecenoliakocTn = 1000
Kon. ao6imaeniedi = 0,005
Kon. aofimaenciirasa = 0,0003
Mnomocrs vedm = 085
Amocheprei aasnern = 1





fig.2. Data entry window
The results of these experiments are shown in text (Fig. 3) and graphical modes (Fig. 4).
[image: image28.png]=50 ALF=1.01KP=0.4MP=0.2 PAT-
‘QN=0.0005 QG=0.0003 QJ=1000
RN=0.85R=5.317=273 Z=1MN=3.0
EPS=0.0001

641678 AGN= 155574





fig.3. Results output in text mode
[image: image29.png]DxYangi1 Natjanat_10,689.txt
Dx\Yangi1\Natja\nat_20,775.6xt
D:\angit Natia\nat_30,861.bxt
DxYangi1\Natjanat_40,947.6xt

D:\angit Watia\nat_51,033.bxt

Ains pasriere sbepire niobe cyTion

oges |
g |-
ogas |-
g |-
ogrs |-
g7 |-
ogss |-
g6 |-
ogss |-
o0gs |-
ogas |-
g |-
g |-
0ga |-
g |-
0g |-
g5 |-
o091 |-

1

®

=

a

a2

44

5

a

Series
Series2
Series3
Seriesd
Seriess





fig.4. Results output in grafik mode
Conclusion
The analysis of obtained results of numerical experiments is showing that for the various time intervals and the same intensity of extraction the pressure drop in the well is faster with filtering oil than with filtering gas. When we assume the existence of gas in oil composition the fluidity of the mixture increases. The numerical calculations showed that essential parameters influencing the development technology of hydrocarbon production from the reservoir systems are filtration coefficient and structure of porous rocks. Dynamic layer pressure redistribution depends essentially on the length of the reservoir. With the increasing of length of reservoir the pressure in the well and adjacent points is reduced. 

The software can be used by specialists and organizations that are engaged in the production of hydrocarbons, in order to improve the efficiency of fields.

REFERENCES
1. Chraibi M., Zaleski S., Franco F. Modeling the solution gas drive process in heavy oils // Записки Горного института.– Санкт-Петербург, 2008. – Т. 174. – С. 36-40.

2. Shelukhin V.V. Problem of Capillary Displacement for One Model of Three-Phase Filtration // Journal of Applied Mechanics and Technical Physics. – Novosibirsk, 2003. – Vol. 44. – № 6. – PP. 95-106.
3. Demyanov A. Yu., Dinariev O. Yu., Ivanov E. N.  Simulation of the transfer of water with a fine-disperse gas phase in porous media // Journal of Engineering Physics and Thermophysics . – Minsk, 2012. – Vol. 85.  – № 6. – PP. 1145-1154.

4. Vasilev Yu.N. Automated system for managing the development of gas fields. – M.: Nedra, 1987. –141 p.

5. Charny I. A. Underground Fluid Dynamics. – M.:Gostoptekhizdat, 1963. –396 p.

6. Filinov M.V., Rokhlin I.M. To study the polyphase filtering fluid in a porous medium // Oil-gas and underground hydraulic gas dynamics. – M.: Nedra, 1969. – Issue. 74. – PP. 37-45.
7. Kurbonov N.M. Mathematical model of the multiphase mixture filtration in porous media // Perspectives for the development of information technologies ITPA-2014 : transactions of the international scientific conference. - Tashkent, 2014. - PP. 171-174.
8. Садуллаев Р., Сиддиков А.М., Ирматов Э.К., Абилкасимов Б., Агзамов А. Моделирование функционирования пластовой фильтрационной системы с учетом динамики переходных процессов - Т.: НПО Кибернетика,1991. - 24 с.
1

_1443773307.unknown

_1472047891.unknown

_1478351704.unknown

_1478785032.unknown

_1480152391.unknown

_1478418931.unknown

_1478351426.unknown

_1478351547.unknown

_1478351566.unknown

_1478351446.unknown

_1472047905.unknown

_1443773329.unknown

_1472047878.unknown

_1443773321.unknown

_1443768156.unknown

_1443768326.unknown

_1443768451.unknown

_1443768468.unknown

_1443769251.unknown

_1443768378.unknown

_1443768232.unknown

_1443767978.unknown

_1443768009.unknown

_1443767924.unknown

_1443767971.unknown

